Overall Content

The data (monkeydata.mat) contains spike trains recorded from 98 neural units while the monkey reached 100

times along each of 8 different reaching angles, as well as the monkey’s arm trajectory on each

trial. On each trial, both the neural data and arm trajectory are taken from 300 ms before

movement onset until 100 ms after movement end. There are 100 trials per reaching angle.

(1) .mat File

The .mat file has a single variable named trial, which is an array of dimensions 100(trials) x

8(reaching angles). The trial array contains 100*8 struct data structures which contain 3 fields:

trialId, spikes, handPos. The first is the trial ID number and is therefore a of little consequence.

The latter two are arrays containing the neural data and the hand position traces respectively.

(2) Neural Data

The spike train data takes the form of a time-discretized sequence of zeros and ones with time

steps of 1 ms. Each value in the array therefore takes the binary form 0/1 and represents the

presence or absence of a spike in its neural unit in the 1 ms time window, or bin. A zero

indicates that the unit did not spike in the 1 ms bin, whereas a one indicates that the unit spiked

once in the 1 ms bin. Thus, a spike train of duration T ms is represented by a 1*T vector. The

spike train recorded from the ith unit on the nth trial of the kth reaching angle is accessed as:

trial(n,k).spikes(i,:) where i = 1, . . . , 98, n = 1, . . . , 100, and k = 1, . . . , 8.

(3) Trajectory Data

In this task, the monkey reached to targets on a fronto-parallel screen. Most of the arm

movement was in the plane of the screen along the horizontal directions: handPos(1,:) and

handPos(2,:). The movement perpendicular to the plane of the screen, handPos(3,:), was relatively

small. The three-dimensional arm trajectory recorded on the nth trial of the kth reaching angle is

contained in trial(n,k).handPos, which is a 3*T matrix of the hand position (in mm) at each 1 ms

time step. On each trial, the data in spikes and handPos are aligned in time. The indices k =

1, . . ., 8 correspond to the reaching angles (30/180π, 70/180π, 110/180π, 150/180π, 190/180π,

230/180π, 310/180π, 350/180π) respectively. The reaching angles are not evenly spaced around

the circle due to experimental constraints that are beyond the scope of this problem set.

Note: The neural data have been generously provided by the laboratory of Prof. Krishna Shenoy at

Stanford University.